ProSekKa – Prognose der Sonneneinstrahlung mit einer kostengünstigen Kamera

MSE - Masterthesis

im Studiengang
Elektro- und Informationstechnik

vorgelegt von

Attila Horvath

am 01.02 2019 
an der Hochschule Luzern Technik & Architektur

Advisor: 	Prof. Dr. Klaus Zahn
Coadvisor: 	Dipl. El. Ing. Andreas Rumsch

# Ehrenwörtliche Erklärung

Hiermit erkläre ich, dass die vorliegende Arbeit mit dem Titel: „ProSekKa – Prognose der Sonneneinstrahlung mit einer kostengünstigen Kamera“, selbstständig und ohne fremde Hilfe verfasst und keine anderen als die angegebenen Hilfsmittel benutzt habe. Die Stellen der Arbeit, die dem Wortlaut oder dem Sinn nach anderen Werken entnommen wurden, sind unter Angabe der Quelle kenntlich gemacht. Die Arbeit ist noch nicht veröffentlicht oder in anderer Form als Prüfungsleistung vorgelegt worden.

Horw, den 4.Dezember 2018 														Attila Horvath

# Kurzfassung

Im Mai 2017 wurde im Rahmen der Energiestrategie 2050, der Schrittweise Ausstieg aus der Atomenergie, durch das Schweizer Volk beschlossen. Zukünftig sollen vermehrt, erneuerbare Energien gefördert werden, wie Wasser und Windkraft sowie Sonnenenergie.

Motiviert durch, fallende Preise und steigender Effizienz, nimmt die Anzahl der Photovoltaikanlagen in privatem Besitz, kontinuierlich zu. So betrug die Photovoltaik-Leistung Ende 2016 ca. 1.5 GW was einer Gesamtfläche von rund 11 Km2 entspricht.

Schnell vobeiziehende Wolkenfelder erzeugen hohe kurzfristige Schwankungen der Einstrahlung, typisch im Bereich weniger Skunden. Die so entstehenden schnellen Änderungen der Leistungsabgabe von PV-Analagen können kritische Rückwirkungen auf das elektrische Versorgungsnetz zur Folge haben.

Es werden unterschiedliche Methoden und Modelle zur Vorhersage von Sonnenstrahlung für verschiedene Zeithorizonte verwendet. Insbesondere werden häufig physikalisch Modelle, basierend auf Staelitenbeobachtung, zur Vorhersage eigesetzt. Solche Vorhersagen verfügen jedoch nicht über die räumliche und zeitliche Aüflösung, um Kurzfristprognosen im Bereich von wenigen Minuten zu erstellen, da die genaue Position und Struktur der Wolken nur ungenügend genau vorhergesagt werden kann.

Total Sky Imager oder zu Deutsch Wolken-Kameras, bieten die geforderte räumliche und zeitliche Auflösung für Kurzfristprognosen von 5 bis 30 Minuten. Mit den Mitteln der Bildverarbeitung können Wolken erkannt, Richtung und Geschwindigkeit bestimmt, sowie die momentane Sonneneinstrahlung berechnet werden.

Ziel der hier vorgestellten Arbeit ist die Entwicklung einer preisgünstigen Wolkenkamera, zur kurzzeitigen Vorhersage der Sonneneinstrahlung.

Schlagwörter: Wolken Kamera, HDR Bilder, Vorhersage der Sonneneinstrahlung, Wolken Position, 
Wolken Identifikation, optischer Fluss, Kamera Kalibrierung, Raspberry Pi

# Abstract

In May 2017, as part of the Energy Strategy 20150, Switzerland decided for a phased withdrawal from nuclear energy. In the future, renewable energies such as water and wind power as well as solar energy will be increasingly promoted.

Motivated by falling prices and increasing efficiency, the number of privately owned photovoltaic systems is steadily increasing. At the end of 2016, the photovoltaic capacity amounted to approx. 1.5 GW, which corresponds to a total area of around 11 Km2.

Fast moving cloud fields generate high short-term fluctuations in the radiation, typically in the range of a few seconds. The resulting rapid changes in the power output of PV systems can have severe effects on the electric power grid system. In particulary, satelite images and physics based models are commonly used for prediction. However, such predictions do not have the spatial and temporal resolution to produce short-term forecasts in the range of a few minutes, since the exact position and structure oft the clouds can not be    predicted with the accuracy needed.

Images from total sky cameras, can provide the required spatial and temporal resolution for short-term forecasts from 5 to 30 minutes. By means of image processing, clouds can be detected and tracked. Knowing their direction and speed, as well as the suns position, present solar irradiance can be determined.

Aim of the presented work is the development of a low-cost total sky-imager for short-term solar irradiance forecasts.

Keywords: total sky imager, HDR imaging, irradiance forecasting, cloud tracking, cloud identification, optical flow, camera calibration, raspberry pi

# Inhaltsverzeichnis

Ehrenwörtliche Erklärung	2

Kurzfassung	3

Abstract	3

Inhaltsverzeichnis	4

Abbildungsverzeichnis	6

Tabellenverzeichnis	6

Abkürzungsverzeichnis	7

Vorwort	8

1	Überblick	9

2	Ziele	11

3	Solare Strahlung	12

3.1	Strahlungsangebot der Sonne	12

3.2	Extraterrestrische Strahlung	12

3.3	Spektrale Eigenschaft der Sonnenstrahlung	12

3.4	Atmospherische Interaktion	13

3.4.1	Rayleigh scattering	13

3.4.2	Aerosol scattering and Absorption	14

3.4.3	Gas and Absorption	14

3.5	Direct Normal Irradiance	14

3.6	Global horizontal Irradiance	15

3.7	Diffuse Irradiance	16

3.8	Solarmessgeräte	17

3.9	Wolken	20

3.10	Dreidimensionale Effekte in der Kurzeit Vorhersage	20

3.11	Ursache und Wirkung der PV-Variabilität	20

3.12	Optik und bildgebende Techniken	21

3.12.1	Für die Auswahl der Optik bestimmende Faktoren	21

3.12.2	Bildwinkel	21

3.12.3	Optische Verzerrung	22

3.12.4	Die Verwendung eines Fischaugenobjektivs	22

3.13	Bildverarbeitung	22

3.13.1	Eigenschaften einer Weitwinkelaufnahme	22

3.13.2	CCD	23

4	Übersicht solarer Vorhersage Methoden	24

4.1.1	Klassifikation solarer Vorhersage Methoden	24

5	Risikoanalyse	25

6	Stand der Technik	27

Anhang A: Beispiele für die Gliederung von Abschlussarbeiten	28

A.1 Literaturarbeiten	28

A.2 Systementwicklungen	28

Anhang B: Formatvorlagen	29

Glossar	30

Quellenverzeichnis	31

Stichwortverzeichnis	32

# Abbildungsverzeichnis

Abbildung 1: Verbinden eines Textteils mit einer Formatvorlage	23

Abbildung 2: Verschiedene Schriftarten	24

Abbildung 3: Einfügen von Beschriftungen	26

Abbildung 4: Einfügen eines Querverweises auf eine Abbildung	27

Abbildung 5: Formatierung von Nummerierungen, Aufzählungen und Einrückungen	30

# Tabellenverzeichnis

Tabelle 1: Unterschiede zwischen Abbildungen und Tabellen	27

Tabelle 2: Beispiele für Überschriftebenen	32

Tabelle 3: Aufstellung der wichtigsten Formatvorlagen der Dokumentvorlage	43

# Abkürzungsverzeichnis

Bild bezogen

HDR	high dynamic range

RGB	rot, grün, blau - Farbraum

Verschiedenes

HSLU	Hochschule Luzern

# Vorwort

Das iHomeLab der Hochschule Luzern ist Schweizer Denkfabrik und Forschungszentrum für Gebäudeintelligenz. Der Schwerpunkt der Forschung am iHomeLab liegt in den beiden Themenbereichen «Smart Energy Management» (SEM) und „Ambient Assisted Living“ (AAL). In beiden Bereichen werden mittels Netzwerken verschiedener Sensoren Daten vom Gebäude, den Bewohnern und der Umgebung gesammelt, um damit das Gebäude energieeffizienter, sicherer und komfortabler, oder anders gesagt, intelligenter zu machen.

Eine zunehmend wichtige Herausforderung ist der Umgang mit elektrischer Energie, insbesondere aus neuen erneuerbaren Energiequellen wie PV-Anlagen oder Windgeneratoren. Die aus diesen Quellen erzeugte elektrische Energie fällt nicht gleichmässig an, sondern unterliegt grossen, wetterbedingten Schwankungen. Die Sonnenenergie verändert sich zusätzlich während des Tages- und des Jahresverlaufs.

Mit der Annahme der Energiestrategie 2050 im Mai 2017 durch das Schweizer Volk ist der Ausstieg aus der Atomenergie und die Förderung der erneuerbaren Energien beschlossen worden. Es wird künftig also einen breiteren Energiemix geben, bei dem neben den neuen Erneuerbaren auch die Wasserkraft eine grosse Rolle spielen wird. Am iHomeLab forschen wir an einem intelligenten Umgang mit der elektrischen Energie. Dabei fokussieren wir auf die Energie aus PV-Anlagen und suchen Strategien, wie der Solarstrom optimal genutzt werden kann. Dabei spielt nicht nur die Eigenverbrauchsoptimierung eines Anlagenbesitzers eine Rolle. Die Optimierung muss auch auf Ebene Quartier bis hin zum Versorgungsgebiet eines EVU betrachtet werden.

Für die Optimierung des Verbrauchs von Solarstrom ist die Kenntnis der zukünftigen Wetterentwicklung vorteilhaft. Denn so kann ein System zur Verbrauchsoptimierung die Wetterentwicklung in die Entscheidung mit einbeziehen, ob ein Haushaltgerät ein- oder ausgeschaltet werden soll. Das ist besonders bei Haushaltgeräten wichtig, die einen Aufwärmvorgang beinhalten, z.B. Waschmaschinen. Wenn das System weiss, dass in der nächsten Zeit die Sonne scheinen wird, kann es die Waschmaschine starten. Andernfalls sollte es damit noch zuwarten, bis dann die Sonne genügend Energie liefern wird.

Zur Vorhersage des Wetters für die nächsten Minuten und Stunden sind die verfügbaren Wetterdaten zu unpräzise. Besonders bei wechselnder Bewölkung liefern die öffentlichen Daten keine zuverlässigen Angaben, wann und wie lange die Sonne durch die Wolken drückt. Optimal wäre dazu ein Gerät, welches vor Ort die Vorhersage auf der Basis von lokalen Beobachtungen liefern könnte. Ein solches Gerät gibt es (http://www.fulcrum3d.com/index.php/cloudcam/technology/), ist aber auf den professionellen Einsatz von Wetterdiensten ausgerichtet. Dieses Produkt ist nicht geeignet für die Eigenverbrauchsoptimierung in Eigenheimen, da die Kosten zu hoch sind.

In dieser Master Thesis soll deshalb eine kostengünstige Variante eines «Kurzzeitwettervorhersagers» entwickelt werden. Das Gerät soll über handelsübliche Komponenten verfügen (z.B. RaspberryPi mit einer Webcam), um die Kosten tief zu halten. Als Ergebnis soll die Intensität der Sonneneinstrahlung für die unmittelbare Zukunft zur Verfügung stehen. Diese Information kann anschliessend ein System zur Optimierung des Eigenverbrauchs verwenden, um darüber zu entscheiden, welche Haushaltgeräte gestartet werden sollen.

# Einleitung

## Motivation

Das iHomeLab der Hochschule Luzern ist Schweizer Denkfabrik und Forschungszentrum für Gebäudeintelligenz. Der Schwerpunkt der Forschung am iHomeLab liegt in den beiden Themenbereichen «Smart Energy Management» (SEM) und „Ambient Assisted Living“ (AAL). In beiden Bereichen werden mittels Netzwerken verschiedener Sensoren Daten vom Gebäude, den Bewohnern und der Umgebung gesammelt, um damit das Gebäude energieeffizienter, sicherer und komfortabler, oder anders gesagt, intelligenter zu machen.

Eine zunehmend wichtige Herausforderung ist der Umgang mit elektrischer Energie, insbesondere aus neuen erneuerbaren Energiequellen wie PV-Anlagen oder Windgeneratoren. Die aus diesen Quellen erzeugte elektrische Energie fällt nicht gleichmässig an, sondern unterliegt grossen, wetterbedingten Schwankungen. Die Sonnenenergie verändert sich zusätzlich während des Tages- und des Jahresverlaufs.

Mit der Annahme der Energiestrategie 2050 im Mai 2017 durch das Schweizer Volk ist der Ausstieg aus der Atomenergie und die Förderung der erneuerbaren Energien beschlossen worden. Es wird künftig also einen breiteren Energiemix geben, bei dem neben den neuen Erneuerbaren auch die Wasserkraft eine grosse Rolle spielen wird. Am iHomeLab forschen wir an einem intelligenten Umgang mit der elektrischen Energie. Dabei fokussieren wir auf die Energie aus PV-Anlagen und suchen Strategien, wie der Solarstrom optimal genutzt werden kann. Dabei spielt nicht nur die Eigenverbrauchsoptimierung eines Anlagenbesitzers eine Rolle. Die Optimierung muss auch auf Ebene Quartier bis hin zum Versorgungsgebiet eines EVU betrachtet werden.

Für die Optimierung des Verbrauchs von Solarstrom ist die Kenntnis der zukünftigen Wetterentwicklung vorteilhaft. Denn so kann ein System zur Verbrauchsoptimierung die Wetterentwicklung in die Entscheidung mit einbeziehen, ob ein Haushaltgerät ein- oder ausgeschaltet werden soll. Das ist besonders bei Haushaltgeräten wichtig, die einen Aufwärmvorgang beinhalten, z.B. Waschmaschinen. Wenn das System weiss, dass in der nächsten Zeit die Sonne scheinen wird, kann es die Waschmaschine starten. Andernfalls sollte es damit noch zuwarten, bis dann die Sonne genügend Energie liefern wird.

Zur Vorhersage des Wetters für die nächsten Minuten und Stunden sind die verfügbaren Wetterdaten zu unpräzise. Besonders bei wechselnder Bewölkung liefern die öffentlichen Daten keine zuverlässigen Angaben, wann und wie lange die Sonne durch die Wolken drückt. Optimal wäre dazu ein Gerät, welches vor Ort die Vorhersage auf der Basis von lokalen Beobachtungen liefern könnte. Ein solches Gerät gibt es, ist aber auf den professionellen Einsatz von Wetterdiensten ausgerichtet. Dieses Produkt ist nicht geeignet für die Eigenverbrauchsoptimierung in Eigenheimen, da die Kosten zu hoch sind.

## Zielsetzung

In dieser Master Thesis soll eine kostengünstige Variante eines «Kurzzeitwettervorhersagers» entwickelt werden. Das Gerät soll aus handelsübliche Komponenten z.B. RaspberryPi mit einer Webcam aufgebaut sein, um die Kosten tief zu halten. Als Ergebnis soll die Intensität der Sonneneinstrahlung für die unmittelbare Zukunft zur Verfügung stehen. Diese Information kann anschliessend ein System zur Optimierung des Eigenverbrauchs verwenden, um darüber zu entscheiden, welche Haushaltgeräte gestartet werden sollen.

Funktion
Das Gerät liefert die Strahlungsintensität zurück, welche im Verlauf der nächsten 60 Minuten zu erwarten ist. So kann abgeschätzt werden, wieviel Energie zu welcher Zeit verfügbar sein wird. Der Zeithorizont Vorhersage ist auf Machbarkeit hin zu prüfen. Das Gerät soll möglichst ohne Konfiguration und ohne Lernvorgang funktionieren, so dass es sofort nach Inbetriebnahme funktioniert.

Kommunikation
Die Vorhersagen sollen von einem Optimierungssystem über Standard-Internetprotokolle abgerufen werden können. Die Übertragung der Daten kann drahtlose oder drahtgebundene erfolgen.

Energieversorgung

Der Energieverbrauch muss so gering wie möglich sein und soll eine durchschnittliche Leistungsaufnahme von 2 Watt nicht übersteigen. Das Gerät muss nicht für Batteriebetrieb ausgelegt werden.

Sensorik

Die Anforderungen an die Sensorik sind zu definieren, u.a. die Dynamik, welche ein Sensor umfassen muss. Passende Sensoren zur Beobachtung des aktuellen Wettergeschehens sind zu evaluieren. Naheliegend ist eine Kamera, es soll aber geprüft werden, ob weitere Sensoren in Frage kommen.

Baugrösse und Kosten
Die Baugrösse ist nicht kritisch, soll trotzdem so gering wie möglich gehalten werden und nicht wesentlich über die Grösse einer Zigarettenschachtel hinausgehen.

Die Materialkosten für den Prototyp sollen 50 Franken nicht übersteigen. Es ist aufzuzeigen, welche Kosten für ein Seriengerät zu erwarten sind.

Entwicklung und Feldtest
Die vom Gerät zur Verfügung gestellten Resultate sollen auf einem Webinterface dargestellt werden.  In einem Feldtest ist die Zuverlässigkeit der Vorhersage zu evaluieren.

Schwierigkeiten und Herausforderungen
Sollen Fehlerquellen und Problematiken aufgezeigt werden, die sich wärend dem Aufbau und dem Betrieb der Kameras ergaben. Zudem soll auf den Umgang mit grossen Datenmengen und den damit verbundenen Schwierigkeiten eingegangen werden.

Optimierung

Möglichkeiten zur Erhöhung der Zuverlässigkeit aufzeigen, nach Möglichkeit auch messtechnisch nachweisen.

Möglichkeiten der Kostenoptimierung aufzeigen, z.B. wie weit kann die Auflösung oder der Dynamikumfang der Kamera reduziert werden, um noch zuverlässige Ergebnisse zu erhalten?

# Solare Strahlung

Das Vakuum des Weltalls lässt die Sonnenstrahlen ungebremst bis zur oberen Atmosphäre der Erde gelangen. Erst beim Eintritt in die Erdatmosphäre wird die Sonnenstrahlung durch unterschiedliche Faktoren abgeschwächt.  Dazu gehören Reflexionen an der Atmosphäre oder die Absorption von unterschiedliche Gase in der Atmosphäre. Zudem erhöhen Rayleigh und Mie-Streuung weiter die Reduktion der Bestrahlstärke.

Rayleigh-Streuung entsteht durch Reflexionen des Lichts an den molekularen Bestandteilen der Luft. Dabei nimmt der Einfluss der Rayleigh-Streuung mit abnehmender Wellenlänge des Lichtes zu. Die Mie-Streung dagegen durch wesentlich grössere Partikel, wie zum Beispiel Staub, verursacht. Mie-Streuung ist deshalb von geographischen Gegebenheiten abhängig, wie zum Beispiel Höhe über Meer oder dem Grad der lokalen Luftbelastung.

Ein längerer Weg durch die Atmosphäre, unabhängig von der Zusammensetzung, führt zu einer grösseren Dämpfung der Bestrahlstärke. Dabei beschreibt die sogenannte Air Mass (AM) in welchem Verhältnis, der kürzest mögliche Weg, in der Erdatmosphäre, zum tatsächlichen steht. Ist die Sonne im Zenit, dann beträgt die Air Mass eins und im Weltall null.

## Strahlungsangebot der Sonne

Buch: Solar and infrared measurements Kap. 2.11 Seite 25 unten
Buch Regenerative Energietechnik Kap 3.1 [Viktor Wesselak] Strahlungsangebot der Sonne

Die Sonne wird zu den mittelgrossen Sternen gezählt. In ihrem Innern, finden Kernfusionprozesse statt, bei welcher Wasserstoffatome zu Helium kombiniert werden. Dabei wird elektromagnetische sowie Materiestrahlung freigesetzt. Der Energietransport vom Kern nach Aussen erfolgt zunächst durch Strahlung, insbesondere im Röntgen- und γ-Bereich des Spektrums, d.h. über sehr energiereiche Photonen [ ].

Die Schwerkraft hält die Sonne davon ab, auseinander zu blasen, und der nukleare Fusionszyklus hat die Sonne strahlende Energie in den Raum gebracht ziemlich konstante Rate für Milliarden von Jahren Die effektive Temperatur von

die Sonne ist 5778 K (5505 ° C oder 9941 ° F) Das bedeutet, dass die Sonne ähnlich strahlt, wenn auch nicht genau wie ein schwarzer Körper auf 5778 K. Die Erde ist in einer leicht elliptischen Umlaufbahn mit einer durchschnittlichen Trennung von 149.598.106 km (92.955.888 Meilen) von der Sonne Da die Strahlungsenergieintensität proportional zur Umkehrung der von der Quelle quadrierten Distanz abnimmt, ist die Erdoberflächentemperatur viel kühler als die Sonne mit einer mittleren Temperatur von etwa 287 K ( 14 ° C oder 57 ° F) Beachten Sie, dass die Erde ohne den Treibhauseffekt der Erdatmosphäre noch kühler wäre.

## Extraterrestrische Strahlung

Buch: Solar and infrared measurements Kap. 2.3 Seite 25

https://tu-dresden.de/bu/umwelt/geo/ipf/fern/studium/tutorial/atmosphaerische-korrekturen/folder-2008-04-07-5554546418/2008-10-28.002004101

Extraterrestrische Strahlung oder Solarkonstante nennt man den außerhalb der Erdatmosphäre gemessenen Strahlungsstrom. Er beträgt S0 = 1370 W/m² und ist über längere Zeit betrachtet konstant.

Solar radiation incident just above the earth’s atmosphere is called extraterrestrial radiation  On average the accepted extraterrestrial irradiance is 1366 watts/meter2 (Wm−2)  The solar output, called the total solar irradiance (TSI), varies by about 0 1% in proportion to the changing number of sunspots over the 11-year solar cycle  The term TSI is now used to refer to the extraterrestrial radiation instead of the previ-

ously used term solar constant (I0), an acknowledgment of the very slight variation in the sun’s output  The absolute accuracy to which the TSI can be measured is around ±0 5% (Fröhlich, 2006)

The eccentricity of the earth’s elliptical orbit is 0 0167 and results in the earth being closest to the sun (perihelion) in early January and farthest from the sun (aphelion) in early July  The earth and the sun are 1 67% closer at perihelion and 1 67% farther  apart  at  aphelion  than  their  mean  separation   Since  the  solar  intensity  is inversely proportional to the square of the distance from the sun, the extraterrestrial

irradiance impinging on the earth varies over a range of 6 7% as the earth orbits the sun  The extraterrestrial irradiance outside the earth’s atmosphere on a surface normal to the sun is  DNIo = Io ⋅(Rav /R)2[Wm−2 ]   (2 1)where Rav is the mean sun–earth distance, and R is the actual sun–earth distance

depending on the day of the year  An approximate equation for (Rav/R)2 is where  β  = 2πd/365 radians, and d is the day of the year (Spencer, 1971)  For example, January 15 is the fifteenth day of the year, and February 15 is the forty-sixth day of the year  Since there are 366 days in a leap year, there is a slight difference in the calculation in leap years  To calculate the extraterrestrial global horizontal irradiance, DNIo is multiplied by the cosine of the solar zenith angle (see Figure 2 6)

## Spektrale Eigenschaft der Sonnenstrahlung

Buch: Solar and infrared measurements Kap. 2.11 Seite 42

The ultimate source of the sun’s radiative energy is the fusion of hydrogen atoms in the sun’s core  This energy propagates to the surface through a series of absorption and re-emission processes finally reaching the visible part of the sun’s surface, the photosphere, after tens of thousands of years  The spectral distribution of the sun’s radiation is similar to that of a black body that is heated to the temperature of 5778 K  That is, the amount of radiation varies with the wavelength as shown in Figure 2 14  The majority of solar irradiance is between 300 nm and 3,000 nm  The absorption of some solar radiation by the ions in the chromosphere of the sun results in absorption lines in the extraterrestrial solar spectrum  The effect of these ions can be seen in the solar spectrum in Figure 2 14, especially for wavelengths  below about 700 nm The  spectral  distribution  of  incident  solar  radiation  spans  a  large  range  of wavelengths  from  the  ultraviolet  to  the  infrared  with  the  peak  around  500  nm  Approximately 47% of the incident extraterrestrial solar radiation is in the visible wavelengths from 380 nm to 780 nm  The infrared portion of the spectrum with wavelengths greater than 780 nm accounts for another 46% of the incident energy

and the ultraviolet portion of the spectrum below 380 nm accounts for 7% of the extraterrestrial solar radiation As the sunlight passes through the atmosphere, a large portion of the ultraviolet  (UV) radiation is absorbed and scattered  Air molecules scatter the shorter wave- lengths more strongly than the longer wavelengths  On days with few clouds, the preferential scattering of blue light by air molecules is the reason the sky appears blue  Water vapor and atmospheric aerosols further reduce the amount of direct sun-

light passing through the atmosphere  The information in Table 2 1 shows that on a clear day approximately 76% of the extraterrestrial direct normal irradiance passes through the atmosphere without being scattered or absorbed  The magnitude of the scattering and absorbing are about equal under these idealized conditions Many  molecules  selectively  scatter  or  absorb  radiation  in  specific  wavelength regions, creating large variations in the solar spectrum as measured at the earth’s surface (Figure 2 15)  The abundance of the atmospheric constituents along the sun-light’s path determines the depth of these absorption regions  Therefore, the solar spectrum depends on the length of the path through the atmosphere and the density of the molecules along this path  Said another way, the atmosphere is a highly vari-

able filter affecting the amounts of solar irradiance reaching the surface at any given time and place.

## Atmospherische Interaktion

Buch: Solar and infrared measurements Kap. 12.3 Atmospheric Interactions Seite 229

When  solar  radiation  passes  through  the  atmosphere,  complex  interactions  oc-cur depending on the wavelength of the radiation and the composition of the atmosphere at the time  More specifically, solar radiation either passes unscathed to the surface, or it is scattered or absorbed by molecules, aerosols, cloud

water droplets, and ice crystals Jan Kleissel Solar Energy Forecasting S57 Kap 3.3.2 Aerosols

### Rayleigh scattering

Molecular scattering is the elastic scattering of solar radiation by the molecules in the atmosphere first explained by Lord Rayleigh (1871)  Many papers have been written describing the scattering of solar radiation by molecules, but it is not a dead subject (Eberhard, 2010)  Bodhaine, Wood, Dutton, and Slusser (1999) thoroughly examined the problem of calculating the molecular scattering optical depth as a function of wavelength  Optical depth, in general, is a measure of the wavelength dependent extinction (by scattering or absorption) that occurs as a beam of radiation propagates through a medium  It can be defined using  I(λ)/I0 (λ) =e−τ(λ)m   (12 1)

where I0 is the strength of the source (e g , the spectral solar irradiance) before it enters the atmosphere, I is the strength after it has passed through the atmosphere to the surface, τ is the optical depth in a vertical path, m is the amount of atmosphere traversed relative to the vertical path, and λ is wavelength  A good approximation to optical depth by Rayleigh scattering is given by Hansen and Travis (1974):

P

R P0 0 008569 (1 0 0113 0 00013 )

τ = λ−4 + λ−2 + λ−4   (12 2)

where wavelength λ is in μm, P is the pressure at the measurement site within the earth’s atmosphere in kilopascals, and P0 is the standard pressure at sea level equal to 101 325 kilopascals (100 kilopascals = 1000 millibars)  It is evident from examining Equation 12 2 that Rayleigh scattering is most effective in the shortest wavelengths of the solar spectrum and decreases dramatically at longer wavelengths (approxi-

mately as λ–4)  It is also evident that at higher elevations there is less Rayleigh scattering because of lower atmospheric pressure

### Aerosol scattering and Absorption

Buch: Solar and infrared measurements Kap. 12.3.2 Aerosol scattering and absorpzion Seite 246

Aerosols  are  always  present  in  the  atmosphere   They  manifest  themselves  as  the haze often noticed when looking toward the horizon  Generally, aerosols are concentrated  near  the  surface  but  may  appear  in  layers  especially  when  transported into a region from long distances  When volcanic eruptions occur that are strong enough to inject dust and gases into the stratosphere, such as Mount Pinatubo did in 1991, a stratospheric aerosol layer may persist for years from the sulfur gases that are chemically transformed into aerosols (see Figure 4 9)  The dust particles from an eruption, however, are large and therefore are removed from the atmosphere quickly.

### Gas and Absorption

Buch: Solar and infrared measurements Kap. 12.3.3 Gas Absorbtion Seite 248

The dominant gas absorber affecting the spectral distribution of the incident solar irradiance in the solar spectrum is water vapor with several major absorption bands in the near-infrared  For example, Figure 2 15 indicates the strong water vapor absorption band in the global horizontal irradiance (GHI) spectrum centered near 940 nm  This is of particular importance to many solar photovoltaic (PV) collectors since the peak response of crystalline silicon PV is near the strong water vapor band around 940 nm  Ozone is another important absorber in the atmosphere  However, its strongest absorption is below 380 nm in the Hartley–Huggins bands  The Chappuis ozone band is broad but has modest absorption centered near 610 nm  The Wulf bands in the near-infrared are even less absorbing  Other gases such as O2 and CO2 are everpresent but less significant in the sense that they do not remove a large fraction of the total solar radiation  Nitrogen dioxide (NO2) is an important gas when air pollution concentrations are high  This gas produces a reddish-brown tinge to the skylight

## Direct Normal Irradiance

Buch: Solar and infrared measurements Kap. 4.1 Overview of direct normal irradiance Seite 94

Solar radiation that arrives at the earth’s surface having come directly from the sun is defined as direct normal irradiance (DNI)  Even if the sky is clear, DNI is smaller than would be measured at the top of the earth’s atmosphere because DNI has under-gone scattering (by molecules and aerosols) and absorption (by gases and aerosols) within the earth’s atmosphere  If clouds are between the sun and the observer, and they are optically thick, then no direct normal irradiance reaches the earth’s sur-face  The global horizontal irradiance (GHI) observed at the surface is a mixture of DNI that reaches the earth’s surface without being scattered or absorbed and diffuse horizontal irradiance (DHI), the irradiance resulting from molecules, aerosols, and clouds scattering of the DNI  This partitioning is ever changing because the atmosphere is not static  The governing equation is  GHI = DNI ⋅ cos(sza) + DHI   (4 1)

where sza is the solar zenith angle, the angle between the zenith and solar directions  This partitioning is illustrated in Figure 4 1  It is easy to see that the solid dark line GHI is the sum of the direct normal component on the horizontal in gray (the first term in Equation 4 1) plus the dotted line DHI (the second term of the equation)  This is consistent throughout the day until the sun is completely blocked and the first term on the right-hand side in Equation 4 1 goes to zero, leaving GHI equal to DHI between 16:00 and 17:00 and after 17:30 where the solid dark line and the dotted line coincide

## Global horizontal Irradiance

Buch: Solar and infrared measurements Kap. 5.1Introduction to global horizontal irradiance Seite 120

Global horizontal irradiance (GHI) is the total solar flux available from the hemispheric sky dome that is incident on a horizontal surface  By convention global irradiance implies a hemispheric, 2π steradian, field of view on any surface  GHI is the sum of the solar radiation coming directly from the solar disk, also called the direct normal irradiance (DNI) or “beam irradiance,” which is normal to the rays from the sun, projected (Equation 5 1) onto a horizontal plane and the solar radiation coming from all other directions of the sky dome, or “sky irradiance” (Figure 5 1)  The solar radiation coming from all parts of the sky dome other than directly from the sun is called diffuse horizontal irradiance (DHI)  Planar surfaces of any orientation receive solar radiation directly from the sun plus diffuse radiation from the sky and reflected radiation from the ground  Historically, this has been termed total solar radiation on a tilted surface  Those involved with photovoltaic systems refer to this total irradiance as plane of array (POA) irradiance  Studies of the thermal performance of buildings rely on the amount of solar irradiance on various parts of a building envelope and refer to this total irradiance as global irradiance on tilted surfaces  Traditionally,  the  study  of  nonconcentrating  thermal  collectors has  also  relied  on global irradiance available to tilted flat plate collectors  In this book global horizontal irradiance (GHI) will refer to the total solar irradiance on a horizontal surface  When discussing irradiance on a surface oriented at any angle other than horizontal, the irradiance will be referred to as global tilted irradiance GTI Global irradiance is measured by a pyranometer  The root of the word pyrano is  derived  from  the  Greek  “pyr”  meaning  fire  or  heat  and  “ano”  meaning  sky  Therefore, a pyranometer is a meter for measuring heat from the sky  Earlier versions of the pyranometer were referred to as a 180° pyrheliometer, but the terminology was changed to pyranometer to help avoid the confusion with the term pyrheliometer that

## Diffuse Irradiance

Buch: Solar and infrared measurements Kap. 6.1Introduction 178

Diffuse horizontal irradiance (DHI) is defined as the solar irradiance that has been scattered  by  molecules,  aerosols,  and  clouds  in  the  atmosphere  and  received  on  a horizontal surface (refer to Figure 6 1)  Another perspective on diffuse irradiance is that DHI is the skylight portion of the global horizontal irradiance (GHI) remaining after removing the direct normal irradiance (DNI)  DHI is measured by a properly shaded pyranometer, or it can be computed from measurements of DNI and GHI using Equation  4 1   Light  reflected  by  objects  above  the  surface  contributes  to  the  DHI, but light reflected from the horizontal surface is not included  However, the radiation reflected from the surface that is subsequently reflected back to the surface by the atmosphere is included; that is, multiple scattering components are included in DHI A very clear atmosphere gives the sky a deep blue color because of Rayleigh scat-tering of solar radiation by air molecules  (Scattering and absorption of solar radiation will be discussed in more detail in Chapter 12 ) This blue color is produced because air molecules scatter radiation nearly inversely with the fourth power of the wavelength, that is,  scattering ∝λ−4  The sun appears yellow or red near the horizon since Rayleigh scattering preferentially removes the bluest light from the transmitted DNI Aerosol-scattered light depends on the aerosol particle size  For “typical” continental aerosols, the scattering is approximately  ∝λ−1 3  Heavily polluted skies typically have an even larger negative exponent, and they scatter blue light preferentially, but not as strongly, as in Rayleigh scattering  This wavelength dependence for aerosols is most noticeable in extremely polluted conditions where the solar disk appears red even with the sun high in the sky, when the amount of Rayleigh scattering would ordinarily be insufficient to cause noticeable reddening  Although the sun’s disk appears red for these hazy conditions, skylight appears not blue but milky white, especially near the horizon because all photons are scattered multiple times during these heavy pollution episodes  The range of sky color from dark blue to milky blue to milky white is a rough gauge of the amount of aerosol in the atmosphere Cloud particles have large sizes relative to visible wavelengths, and their scattering wavelength dependence is nearly neutral; therefore, they appear to have the white color of the sun  The spatial distribution of skylight is complex and unpredictable for partly cloudy conditions but can be described mathematically for some conditions  Moon and

Spencer (1942) developed a simple and useful approximation of the spatial distribution

Kap 1.3 Solar Power vs Solar Irradiance Seite 8  (Jan Kleissl Solar Energy Forecasting)

Kap 1.4 Direct, Diffuse and Global solar radiation and instrumentation (Jan Kleissl Solar Forecast)

Kap 1.5 Athmosperic properties affecting solar irradiance               (Jan Kleissl Solar Forecast)

Kap 5.51 High-Quality Measurement of DNI, GHI and DHI (Jan Kleissl Solar Forecasting)

## Solarmessgeräte

Kurz und knackick in: Quasching_Regenerative_Energiesysteme S83 Kap 2.8 Solarstrahlungsmesstechniken und Sonnensimulatoren

Zur Messung der globalen Bestrahlungsstärke haben sich zwei Messprinzipien durchgesetzt. Dabei werden entweder Halbleitersensoren oder thermische Sensoren eingesetzt. Messgeräte die auf einem dieser Sensortypen basieren, werden als Pyranometer bezeichnet [1].

### Pyranometer

Bei den Halbleitersensoren handelt es sich um photovoltaische Solarzellen aus Silizium. Der erzeugte Strom im Halbleiter steigt proportional mit der Bestrahlungsstärke der Sonne. Über einen Messwiderstand, lässt sich der Strom in ein Spannungssignal umwandeln. Halbleitersensoren reagieren empfindlich auf Temperaturänderungen, deshalb sollte auch die Temperatur aufgezeichnet werden und gegebenenfalls eine Korrektur der Messwerte vorgenommen werden. Ein weiterer Nachteil von Halbleitersensoren ist, dass nicht alle Wellenlängenbereiche der Sonnenstrahlung gleichermassen erfasst werden. So ist es möglich das bei tiefen Sonnenstand, falsche Werte gemessen werden.

Ein thermischerer Sensor, siehe Abbildung 1, besteht aus einer geschwärzter Empfängerfläche, die durch einen doppelwandigen Glasdom thermisch, von der umgebenden Luft, isoliert ist. Die Sonnenstrahlen durchdringen den Glasdom und erwärmen die Empfängerfläche. Proportional zur zunehmender Bestrahlungsstärke, steigt auch die Temperaturdifferenz zwischen der Aussentemperatur und der Temperatur im Inneren des Glasdoms. Im Gegensatz zum Halbleitersensor zeichnet sich der thermische Sensor, durch konstante spektrale Empfindlich über grosse Teile des Spektrums aus.

Abbildung 1: Links: Pyranometer mit thermischen Sensor, rechts: Pyranometer mit Silizium-Halbleitersensor [1] V. Quaschning: Bild 2.29 [S. 84].

Book: Marius Paulescu Weather Modeling and Forecasting of PV Systems Operation S 34 Kap 2.2.1 Solar Radiometers

### Pyrheliometer

Quasching_Regenerative_Energiesysteme S85 Kap 2.8.2 Messung der direkten und diffusen Bestrahlstärke

The pyrheliometer is a broadband instrument that measures the direct beam component Gn of solar radiation. Consequently, the instrument should be permanently pointed toward the Sun. A two-axis Sun tracking mechanism is most often used for this purpose. The detector is a multi-junction thermopile placed at the bottom of a collimating tube (Fig. 2.3a) provided with a quartz window to protect the instrument. The detector is coated with optical black paint (acting as a full absorber for solar energy in the wavelengths range 0.280–3 lm). Its temperature is compensated to minimize sensitivity of ambient temperature ﬂuctuations. The pyrheliometer aperture angle is 5°. Consequently, radiation is received from the Sun and a limited circumsolar region, but all diffuse radiation from the rest of the sky is excluded. A readout device is used to give the instant value of the direct beam irradiance. Its scale is adapted to the sensitivity of the particular instrument in order to display the value in SI units, Wm-2.

Pyranometer

Pyranometers are broadband instruments that measure global solar irradiance incoming from a 2p solid angle on a planar surface. A typical pyranometer is schematically represented in Fig. 2.4a. It consists of a white disk for limiting the acceptance angle to 180° and two concentric hemispherical transparent covers

made of glass. The two domes shield the sensor from thermal convection, protect it against weather threat (rain, wind, and dust) and limit the spectral sensitivity of the instrument in the wavelength range 0.29–2.8 lm. A cartridge of silica gel inside the dome absorbs water vapor. A pyranometer can be also used to measure the diffuse solar irradiance Gd,  provided that the contribution of the direct beam component is eliminated. For this, a small shading disk can be mounted on an automated solar tracker to ensure that the pyranometer is continuously shaded. Alternatively, a shadow ring may prevent the direct component Gb from reaching the sensor whole day long (see Fig. 2.4b). Because the daily maximum Sun elevation angle changes day by day, it is necessary to change periodically (days lag) the height of the shadow ring.

Sunshine duration measurement

According to (WMO 2008), sunshine duration in a given period is deﬁned as the sum of the time intervals for which the direct solar irradiance exceeds the threshold of 120 Wm-2. In practice, two methods are widely used for measuring sunshine duration—burning card method and pyranometric method—which will be brieﬂy presented next.

Burning card method is based on the Campbell–Stokes sunshine recorder, which basic setup consists of a glass sphere mounted concentrically in a segment of a spherical bowl (Fig. 2.5). The support is adjustable so that the axis of the sphere may be inclined to the angle of the local latitude. The spherical bowl segment holds the recording card. The glass sphere focuses the direct beam solar radiation on to the card, burning a trace whenever the Sun is shining. The position and length of the trace indicate the starting time and duration of the sunshine interval. The errors of this recorder are mainly due to the dependence of burning initiation on card’s temperature and humidity as well as to the overburning effect, especially in case of broken clouds (Kerr and Tabony 2004).

Strahlungsart Beschreibung Messinstrument -> Tabellarische Zusammenstellung gut!
Die folgende Tabelle gibt einen Überblick über Strahlungsarten und mit welchen Instrumenten diese gemessen werden können:

http://www.ammonit.com/de/wind-solar-wissen/solarmessung/473-messung-der-sonnenstrahlung

## Wolken

Jan Kleissel Solar Energy Forecasting S57 Kap 3.3.1 Clouds

## Dreidimensionale Effekte in der Kurzeit Vorhersage

Jan Kleissel Solar Energy Forecasting S70 Kap 3.7.1 Three-Dimesional Effekts in short-Term Forecasting

## Ursache und Wirkung der PV-Variabilität

Jan Kleissel Solar Energy Forecasting S129 Kap 6.1  Solar Resource Variability

In this chapter, we focus on the short-term temporal variability of the solar resource caused by weather and passing clouds, corresponding to timescales of seconds to tens of minutes.

A smal cloud passing in front of the Sun can cause a small PV installation to go from full production to almost none and then back to full production in a matter of secondsdthis impact is of concern to grid operators.

A general measure of the solar resource for nonconcentrating ﬂatplate1 solar-system conﬁgurations is global horizontal irradiance (GHI). Shortterm GHI variability includes the effect of predictable factors due to changes in Sun position and unpredictable factors due to weather/clouds.

Short-term ﬂuctuations and ramp rates of less than 20 s will affect small individual systems, but should be minimized when a ﬂeet of such systems covers an area of a few square kilometers. At the system level, these ﬂuctuations can (rarely) cause localized voltage disturbances and can cause systems to trip ofﬂine. The best way to address them is at the interconnection-hardware level, which can include appropriate “shock absorbers” to increase their electrical inertia and eliminate such risks.

Jan Kleissel Solar Energy Forecasting S129 Kap 7.1  Quantifying and Simulating Solar-Plant Variability Using Irradiance Data

As opposed to conventional power sources such as coal or nuclear power plants, the power output from PV plants is variable. This variability is a concern to grid operators, as unanticipated changes in PV output can strain the grid. At short timescales (seconds), sharp changes in power output can cause local voltageﬂicker issues. At longer timescales (minutes), producing less PV power than expected can cause balancing and, as a result, frequency issues, where load can exceed generation. PV power variability can be counteracted by other, fastramping generation sources (e.g., gas turbines) and by storage systems (e.g.,

batteries), but both are quite expensive and substantially increase plant cost.
The main causes of solar variability are the movement of the Sun through the sky (i.e., power output drops to zero at night) and clouds passing over a PV module, temporarily reducing power output. Both of these effects can be seen in Figure 7.1, where in a coarse sense the output follows the height of the Sun in

the sky, with maximum at solar noon and minimum at sunrise and sunset. In a ﬁner view, however, there are many short ﬂuctuations due to passing clouds or cloud fronts. Other factors, such as atmospheric content, module temperature, and system-speciﬁc conditions can also cause variability in plant output, but

their effects are typically small. While the variability due to Sun movement can be precisely predicted and

causes noticeable changes only over timescales of many minutes to hours, cloud-caused variability is difﬁcult to predict and can cause signiﬁcant changes in output in seconds. Fortunately, though, geographic diversity within a PV plant leads to a reduction in cloud-caused variability, as some modules may be covered by cloud while others see clear sky. This is seen visually in Figure 7.1, as the envelope of ﬂuctuations is smaller for the PV plant than for the single point sensor, showing that relative variability is reduced for the PV plant. The amount of this reduction in variability changes from plant to plant and day to

day, as smoothing depends on plant layout, the timescale of interest, and daily meteorological conditions.

## Optik und bildgebende Techniken

Diverse Arbeiten: All Sky Kam Astro MANGO Report

### Für die Auswahl der Optik bestimmende Faktoren

### Bildwinkel

Diverse Arbeiten: All Sky Kam Astro MANGO Report S27

In the fields of imaging and photography, angle of view describes the angular extent of a given scene that is imaged by a camera. The wider the angle, more angle of the scene is captured and the opposite the holds true. The angle of view of a lens is a function of the focal length of the lens and is inversely proportional to its tangent in normal lenses. Thus, longer the focal length, narrower will be the angle of view. This can be seen in Figure 2.9.

To this point, it might seem that lenses with wider-angle views are better since they capture more information. While this is true, there are consequences of using wide-angle lenses. Wide-angle lenses are prone to optical distortion.

### Optische Verzerrung

Diverse Arbeiten: All Sky Kam Astro MANGO Report S27

In the field of Optics, distortion is an optical aberration where straight lines in a scene do not appear as straight lines in the image. Majority of camera lenses produce images in line with the law of central perspective. This means that relative to the observer, all the converging lines lead towards a single vanishing point at the center of the image. This kind of projection of three-dimensional space onto a two-dimensional image surface is called rectilinear projection or gnomonic projection. The reason this rule is not obeyed, especially in wide-angle optics, is that the image scale is not constant throughout the entire image field. This means that the focal length of a lens showing distortion changes with the distance of an image point from the optical axis [21]. The effect of distortion when using wide-angle optics can be seen in Figure 2.10.

https://de.wikipedia.org/wiki/Bildwinkel

### Die Verwendung eines Fischaugenobjektivs

Diverse Arbeiten: All Sky Kam Astro MANGO Report S28

As solid-state (CCD) imagers improved with technology, new systems have started employing wide-angle optics which can view emissions over large geographic areas. Fisheye lenses are ultra-wide-angle lenses that achieve extremely wide angles of view by losing the straight lines of perspective as seen in rectilinear

images. Figure 2.11 shows a picture take using a fisheye lens. It is worth noting how the image is different from the conventional rectangular image taken from a rectilinear lens, which appears more natural to a human observer. Due to the optical distortion because of fisheye lenses and the effect of undesirable characteristics of all-sky images, post-processing of these images is required. The need for this and the generally applied techniques are discussed in more detail in the following section.

https://de.wikipedia.org/wiki/Weitwinkelobjektiv

## Bildverarbeitung

### Eigenschaften einer Weitwinkelaufnahme

Diverse Arbeiten: All Sky Kam Astro MANGO Report S29

Cameras used for all-sky imaging record rasterized images with a finite number of pixels (picture elements). This means that an image acquired using these cameras have a finite number of dots defined in the horizontal as well as the vertical axis. Since the images are stored as files with pixelwise information, no information about the geographical coordinates is present in the image.

Furthermore, since the images are taken using a fisheye lens, they are not rectilinear. The aforementioned characteristics of the images are a consequence of the imaging technique employed. Apart from these, other effects may be noticeable because of the content of the actual scene. Other systematic errors may also exist in the image which would then need correction. For instance, CCDs are prone to having read-out noise especially when they have faint background levels, which is the noise associated with reading each on-chip amplifier [25]. Figure 2.12 shows how a preprocessed image may appear.

### CCD

Booklet: Photovoltaic and solar Forecasting: state oft the Art; Reports IEA PVPS T14-01:2013 S13

At the University of California San Diego, sky imagers (USIs) have recently been specifically developed for solar forecasting applications and feature high resolution, high dynamic range, high stability imaging chips that enable cloud shadow mapping and solar forecasting at unprecedented spatial detail (Figure 3a).

# Übersicht solarer Vorhersage Methoden

### Klassifikation solarer Vorhersage Methoden

Jan Kleissel Solar Energy Forecasting S166 Kap 8 Overview of solar-Forecasting Methods and a Metric for Accuracy Evaluation

The choice of solar-forecasting method depends strongly on the timescales involved, which can vary from horizons of a few seconds or minutes (intra-hour), a few hours (intraday), or a few days ahead (intraweek). Different time horizons are relevant according to the forecast application.

Short-term irradiance forecasting using skycams: Motivation and developement (Samuel R. West) S 5

The time horizon over which this information can be provided depends on a number of factors including the imager ﬁeld of view, wind speed and computation time. A typical maximum horizon for sky imager forecasts is on the order of 20 min, with update frequencies of between 10 and 30 s. The upper forecast horizon limit is mainly dependent on the amount of time taken for a cloud to move from the edge of the camera’s ﬁeld of view to the ‘danger zone’. This zone describes the region in which a cloud may intercept the sun and shade the target area. It is desirable for the camera to have a wide and unobstructed ﬁeld of view, however it is possible to use multiple cameras and for their images to be ‘stitched’ together in order to extend the forecast horizon.

Booklet: Photovoltaic and solar Forecasting: state oft the Art; Reports IEA PVPS T14-01:2013 S13

The actual look‐ahead time for which this method has significant skill will depend, among other things, upon the cloud velocity and the height of the clouds (the ratio of the cloud velocity to the cloud height defines an angular velocity about the WSI which determines the time duration of the cloud being in the field‐of‐view). For low and fast clouds the forecast horizon may only be 3 minuets while for high and slow clouds it may be over 30 minutes, but generally horizons between 5 to 20 minutes are typical. Even if cloud size and velocity could be determined accurately, the forecast accuracy depends on the rate at which the cloud field is departing from the evolution defined by the cloud motion vectors (i.e. development, dissipation, etc.).

# Risikoanalyse

Jan Kleissel Solar Energy Forecasting S79 Kap 4 Eval of Risk in solar-project financing

Jan Kleissel Solar Energy Forecasting S79 Kap 4.5 Techniques for quantifying and managing resource risks

# Stand der Technik

Die Idee, eine spezielle Word-Dokumentvorlage zum Schreiben von Abschlussarbeiten und anderen wissenschaftlichen Arbeiten zu entwickeln, ist zwar naheliegend, doch ist eine Internet-Recherche nach einschlägigen Lösungen wenig ergiebig

# Anhang A: Beispiele für die Gliederung von Abschlussarbeiten

Die nachfolgenden Gliederungen stellen lediglich Vorschläge dar, die stets am konkreten Fall überprüft und in der Regel angepasst werden müssen.

## A.1 Literaturarbeiten

- Überblick (oder: Zusammenfassung, „Executive Summary“, alles Wichtige für den „Manager“ oder Schnellleser)

- Fragestellung (oder: Ziele, Ausgangspunkt, Motivation)

- Übersicht über den Stand der Wissenschaft und Technik (Beschreibung der Lösungsansätze, Beispiele etc. in einzelnen Abschnitten)

- Bewertung der einzelnen untersuchten Ansätze, Beispiele etc., Identifikation von Defiziten

- Synthese: Erstellung einer Gesamtschau, allgemeine Prinzipien, Beschreibung einer eigenen Sicht auf das Problem, evtl. auch eigene Vorschläge

- Zusammenfassung (Erklärung des Nutzens), Ausblick

Anhang: eventuell recherchierte Texte, Produktbeschreibungen, etc.

## A.2 Systementwicklungen

- Überblick (oder: Zusammenfassung, „Executive Summary“, alles Wichtige für den „Manager“ oder Schnellleser)

- Problemstellung (oder: Ziele, Ausgangspunkt), Vorgesehener Benutzerkreis, Bedürfnisse der Benutzer

- Stand der Technik (Wie wird das Problem bisher gelöst, wo sind die Defizite)

- Gewählter Lösungsansatz (allgemeines Prinzip, welche Werkzeuge, z.B. Programmiersprachen werden verwendet)

- Beschreibung der durchgeführten Arbeiten

- Ergebnis (z.B. Screenshots mit Erläuterungen)

- Zusammenfassung (Erklärung des Nutzens), Ausblick

Anhang: evtl. (ausgewählte) Programmbeispiele

Evtl. CD-ROM als Beilage

# Anhang B: Formatvorlagen

Nachfolgend sind die für die Benutzer der Dokumentvorlage wichtigsten Formatvorlagen aufgelistet. Vermerkt ist jeweils auch der Zweck der Formatvorlage und ob es sich um eine neue oder eine modifizierte Formatvorlage handelt.

Tabelle 3: Aufstellung der wichtigsten Formatvorlagen der Dokumentvorlage

| Formatvorlage | Zweck | neu oder modifiziert |
| --- | --- | --- |
| Abbildung | Abbildungen | neu definiert |
| Beschriftung | Beschriftung von Abbildungen | modifiziert |
| Computerprogramm | Texte von Computerprogrammen u.ä. | neu definiert |
| Fußnotentext | Fußnoten | modifiziert |
| Fußnotenzeichen | Fußnotenzeichen | modifiziert |
| Fußzeile | Fußzeile (normalerweise ausgeschaltet) | modifiziert |
| Kopfzeile | Kopfzeile (wird automatisch erstellt) | modifiziert |
| Literaturverzeichnis | Literaturangaben im Literaturverzeichnis | neu definiert |
| Standard | normaler Fließtext | modifiziert |
| Tabellenüberschrift | Beschriftung von Tabellen | neu definiert |
| Titel | Titel auf der Titelseite | modifiziert |
| Überschrift 1 | Überschrift der Ebene 1 | modifiziert |
| Überschrift 2 usw. bis ... | Überschrift der Ebene 2 | modifiziert |
| Überschrift 9 | Überschrift der Ebene 9 | modifiziert |
| Untertitel | Text auf der Titelseite | modifiziert |
| Zitat | Hervorgehobenes Zitat | neu definiert |

# Glossar

Absatz (engl. paragraph): Absätze gliedern den Fließtext. In dieser Formatvorlage sind Absätze untereinander stets durch einen zusätzlichen Zeilenabstand voneinander getrennt, möglich wären aber auch andere Absatztrennmarkierungen wie z.B. Einrückungen. In Word werden Absatzgrenzen durch sogenannte Absatzmarken festgelegt, die durch einmaliges Betätigen der Enter-Taste eingegeben werden. Mit der Menüfunktion Extras – Optionen – Ansicht können Absatzmarken sichtbar gemacht werden.

Formatvorlage (engl. style): Formatvorlagen dienen zur Formatierung eines Textstückes, meist eines Absatzes. In Formatvorlagen können Texteigenschaften wie Zeicheneigenschaften, Absatzeigenschaften, Tabulatoren, Rahmen, Sprache und Nummerierungen festgelegt werden und in dieser Kombination einem Textstück zugewiesen werden

Dokumentvorlage (engl. style sheet): Dokumentvorlagen sind Gesamtheiten von Formatvorlagen, die erforderlich sind, um einen bestimmten Dokumenttyp (z.B. Diplomarbeit) zu formatieren. Im Textsystem Microsoft Word stellen Dokumentvorlagen einen eigenen Dateityp mit der Dateiendung .dotx dar. Im Kontext dieser Arbeit wird der Begriff Dokumentvorlage etwas weiter gefasst und umfasst auch ein Word-Dokument, das neben einer Definition von Formatvorlagen auch musterhafte Textteile enthält.

# Quellenverzeichnis

HBI (2001): Merkblatt zur Diplomarbeit. Hochschule für Bibliotheks- und Informationswesen Stuttgart.

[1]	V. Quaschning, Regenerative Energiesysteme: Technologie - Berechnung - Simulation, 9., aktualisierte und erweiterte Auflage. München: Hanser, 2015.

# Stichwortverzeichnis

Abbildung  25

Abbildungsverzeichnis  18

Abkürzungsverzeichnis  18

Absatz  35, 44

Absatzmarken  39

Abstand zwischen Absätzen  35

Abstract  18

Angebotsorientiertheit  12

Anhänge  19

Anleitungen  13

Ansicht  37

Arbeitserleichterung  12

Aufzählungen  30

Auto-Wiederherstellen  38

Beschriftung  25

Bildschirmabzüge  25

Bindestrich  36

Computerprogramm  29

Datei-Info  39

Dokumentvorlage  13, 44

Ebenen  31

Eidesstattliche Versicherung  18

Einrückungen  30

Fehler  37

Fließtext  24

Formatvorlage  14, 22, 44

Formatvorlagen-Katalog  33

Funktionalitäten  22

Fußnoten  25

Glossar  20

Index  20

Inhalt der Arbeit  19

Inhaltsverzeichnis  18

Keywords  18

Kopfzeile  23

Kurzfassung  18

Leerzeichen  39

Literaturverzeichnis  33

Muster  14, 16

nicht druckbare Zeichen  20, 36

Nummerierungen  30

Qualitätssicherung  12

Quellenangabe  29

Rechtschreibprüfung  25

Rechtschreibung  16, 35, 39

Schlagwörter  18

Schnellspeicheroption  38

Schriftart  24

Seiteneinrichtung  23

Serifenschrift  24

Sicherungen  37

Sichtbarkeit  36

Silbentrennung  25, 36, 39

Speichern  37

Standard (Formatvorlage)  24

Stichwortverzeichnis  20

Tabellen  27

Tabellenüberschrift  28

Tabellenverzeichnis  18

Titelblatt  17

Trennstriche  36

Überschriften  31

Untertitel  17

Vorgaben  13

Vorwort  19

Word-Dokument  14

Word-Dokumentvorlage  14, 33

Zitat  28

http://www.fulcrum3d.com/index.php/cloudcam/technology